## Worksheet 3.1C

# Sustainability project payback – worked example

This is a worked example of the cost benefit of upgrading a new lighting system. 
Use this as a guide to evaluate the payback on sustainability projects. This is an example only – adapt this worksheet to suit your organisation’s requirements.

| Project name: | Upgrade to new lighting system |  |  |  |
| --- | --- | --- | --- | --- |
|  | Existing fixture |  | New fixture |  |
| Lamp type | Standard fluorescent |  | Triphosphor fluorescent |  |
| 1. Power input |  |  |  |  |
| Number of fittings | 20 |  | 21 |  |
| Number of lamps per fitting | 2 |  | 1 |  |
| Total number of lamps | 40 |  | 21 |  |
| Power input for each lamp | 36 | watts | 36 | watts |
| Power input for each ballast* | 9 | watts | 4 | watts |
| Total power input = total number of lamps x power input of (lamp + ballast) | 1800 = 40 x (36+9) | watts | 756* = 21 x (36+4-4) | watts |
| *Note: an electronic ballast will run a lamp at a lower wattage (i.e. 32 W) but there is still a ballast loss of 4 W. Therefore the power input for one lamp and the ballast = 36 W |  |  |  |  |
| 2. Annual energy consumption by converting power input to kilowatt hours (kWh) |  |  |  |  |
| Operating hours per year | 3000 | hours | 3000 | hours |
| Total annual energy consumption = total power input/1000 x operating hours per year | 5400 = 1800 watts x 3000hr/1000 | kWh | 2268 = 756 watts x 3000hr/1000 | kWh |
| 3. Annual lighting energy costs |  |  |  |  |
| Average electricity price ($ per kWh) | $0.10 |  | $0.10 |  |
| Total annual lighting energy costs = energy cost x total energy consumption | $540 = $0.10 x 5400 | kWh | $227 = $0.10 x 2268 | kWh |
| 4. Annual savings |  |  |  |  |
| Energy saved = total energy consumption of (existing lights – new lights) | 3132 = 5400-2268 | kWh |  |  |
| Any additional hidden savings? |  |  |  |  |
| Is the space air conditioned? If yes, 25% more energy savings can be made. | 3915 = 3132 x 1.25 | kWh |  |  |
| If no, use initial energy savings calculated. |  |  |  |  |
| Total energy to be saved per year | 3915 | kWh |  |  |
| Dollar savings per year = energy cost x total energy savings per year | $391.50 |  | = $0.10 x 3915 |  |
| Annual maintenance cost saved | $50 |  |  |  |
| Total dollar savings per year | $441.50 | kWh |  |  |
| Estimated conversion cost (parts and labour) | $1500.00 |  |  |  |
| 5. Simple payback |  |  |  |  |
| = conversion cost/total annual savings | 3.4 = $1500/$441.50 | years |  |  |
| Annual rate of return (ROR) = total dollar savings per year/conversion cost x 100 | 29% = $441.5/$1500 x 100 |  | A 100% ROR means that the energy savings will pay back the conversion cost within one year. In this case it will be paid back in under a year |  |
| 6. Greenhouse gas reduction | 1 kWh = 1.06 kg CO2, 1 MWh = 1.06 t CO2 |  |  |  |
| = total energy savings/1000 x 1.06 = tonnes of CO2 abated | 4.2 = 3915 kWh/1000 x 1.08 | tonnes |  |  |
| On average, 1 car produces 4.5 tonnes of CO2 in a year | Almost 1 car = 4.2/4.5 | tonnes |  |  |

NSW emission factor (full fuel cycle) for electricity purchased by end users, 
‘National Greenhouse Accounts (NGA) Factors’, Australian Government Department of Climate Change 2008 (www.climatechange.gov.au/workbook/index.html)